Assignment-4

- What is Split and Merge?

Ans: Split and merge segmentation is an image processing technique used to segment an image. The image is successively split into quadrants based on a homogeneity criterion and similar regions are meaning that there is a parent child node relationship. The total region is a parent, and each of the four splits is a child.

Example:  The following example shows the segmentation of a gray scale image using matlab. The homogeneity criterion is thresholding max region – min region < 10 for a region to be homogeneous.

The blocks created during splitting shown in the following picture:

- Explain the Mean Shift Algorithm.

Ans: Mean shift clustering aims to discover “blobs” in a smooth density of samples. It is a centroid-based algorithm, which works by updating candidates for centroids to be the mean of the points within a given region. These candidates are then filtered in a post-processing stage to eliminate near-duplicates to form the final set of centroids.

The following figures explain more about Mean Shift Algorithm:

- Each hill represents a cluster.

- Peak (mode) of hill represents “Center” of Cluster.

- Each pixel climbs the steepest hill within its neighborhood.

- Pixel assigned to the hill (cluster) it climbs.

Given: Distribution of N pixels in feature space.

Task: Find modes (clusters) of distribution.

Clustering:

- Set mi = fi as initial mean for each pixel i.

- Repeat the following for each pixel mi:

- Place window of size W and mi.

- Compute centroid m within the window. Set mi = m.

- Stop if shift in mean mi is less than a threshold E. mi is the mode.

- Label all pixels that have same mode as belonging to same cluster.

3. Write down the steps of Region Split and Merge.

Ans: Apply Region split on following image. Assume that threshold value be <= 4.

Step 1: Identify Max and Min pixel value from the whole image.

Max = 7, Min = 0, Max-Min = 7

7<=4 (False) then Split image

Step 2: Compute for R1a region. Max and Min

Max = 7, Min = 4, Max-Min = 3

3<=3 (True) then No need to Split R1a

Step 3: Compute for R3c region. Max and Min

Max = 3, Min = 0, Max-Min = 3

3<=3 (True) then No need to Split R3c

Step 4: Compute for R2b region. Max and Min

Max = 7, Min = 2, Max-Min = 5

5<=3 (False) then Split R2b into R2b1, R2b2, R2b3, R2b4

Step 5: Compute for R2b1, R2b2, R2b3, R2b4 region. Max and Min.

For all R2b1, R2b2, R2b3, R2b4

Condition have satisfied so no need to splitting.

Step 6: Compute for R4d region. Max and Min

Max = 7, Min = 0, Max-Min = 7

7<=3 (False) then Split R4d into R4b1, R4b2, R4b3, R4b4

Step 5: Compute for R4b1, R4b2, R4b3, R4b4 region. Max and Min.

For all R4b1, R4b2, R4b3, R4b4

Condition have satisfied so no need to splitting.

- Write down Graph based Segmentation.

Ans: Graph partitioning methods are an effective tool for image segmentation since they model the impact of pixel neighborhoods on a given cluster of pixels or pixel, under the assumption of homogeneity in images. In these methods, the image is modeled as a weighted, undirected graph. Usually a pixel or a group of pixels are associated with nodes and edge weights define the similarity between the neighborhood pixels. The graph (image) is then partitioned according to a criterion designed to model "good" clusters. Each partition of the nodes (pixels) output from these algorithms are considered an object segment in the image; see Segmentation-based object categorization. Some popular algorithms of this category are normalized cuts, random walker, minimum cut, isoperimetric partitioning, minimum spanning tree-based segmentation, and segmentation-based object categorization.

Images as Graphs:

- A vertex for each pixel.

- An edge between each pair of pixels.

- Graph Notation: G = (V, E) where V and E are the sets of vertices and edges, respectively.

- Each edge is weighted by the affinity or similarity between its two vertices.

Let i and j be two pixels whose features are fi and fj.

Criteria for Graph Cut:

- A pair of Vertices (pixels) within a subgraph have high affinity.

- A pair of vertices from two different subgraphs have low affinity.

That is, minimize the cost of cut. Also called Min-Cut.

Each subgraph is an image segment.

5. What is Normalized Cut?

- The normalized cut criterion measures both the total dissimilarity between the different groups as well as the total similarity within the groups. We show that an efficient computational technique based on a generalized eigenvalue problem can be used to optimize this criterion.

- To avoid this unnatural bias for partitioning out small sets of points, we propose a new measure of disassociation between two groups. Instead of looking at the value of total edge weight connecting the two partitions, our measure computes the cut cost as a fraction of the total edge connections to all the nodes in the graph. We call this disassociation measure the normalized cut (Ncut):

N cut(A; B) = cut(A; B)/assoc(A; V ) +cut(A; B)/assoc(B; V )

Assignment- 5

1. What is Motion Representation?

- Motion analysis was motivated by the need for tracking an object and advancement in image processing hardware.

- Analyzing human motion is a challenging task with a wide variety of applications in computer vision and in graphics. One such application, of particular importance in computer animation, is the retargeting of motion from one performer to another. While human motions are captured using video, requiring 2D-to-3Dpose and camera recovery, before existing retargeting approaches may be applied.

- In computer vision the motion field is an ideal representation of 3D motion as it is projected onto a camera image. Given a simplified camera model, each point {\display style (y_{1},y_{2})} in the image is the projection of some point in the 3D scene but the position of the projection of a fixed point in space can vary with time.

- The motion field can formally be defined as the time derivative of the image position of all image points given that they correspond to fixed 3D points.

- This means that the motion field can be represented as a function which maps image coordinates to a 2-dimensional vector.

- The motion field is an ideal description of the projected 3D motion in the sense that it can be formally defined but in practice it is normally only possible to determine an approximation of the motion field from the image data.

2. What is Optical Flow?

- Optical flow or optic flow is the pattern of apparent motion of objects, surfaces, and edges in a visual scene caused by the relative motion between an observer and a scene. Optical flow can also be defined as the distribution of apparent velocities of movement of brightness pattern in an image.

- The concept of optical flow was introduced by the American psychologist James J. Gibson in the 1940s to describe the visual stimulus provided to animals moving through the world.

- The term optical flow is also used by robotic cists, encompassing related techniques from image processing and control of navigation including motion detection, object segmentation, time-to-contact information, focus of expansion calculations, luminance, and motion compensated encoding, and stereo disparity measurement.

- Optical flow, or motion estimation, is a fundamental method of calculating the motion of image intensities, which may be ascribed to the motion of objects in the scene. Optical flow is an extremely fundamental concept that is utilized in one form or another in most video-processing algorithms.

3. Explain Feature-based Techniques.

- The method of finding image displacements which is easiest to understand is the feature-based approach. This finds features (for example, image edges, corners, and other structures well localized in two dimensions) and tracks these as they move from frame to frame. This involves two stages.

- Firstly, the features are found in two or more consecutive images. The act of feature extraction, if done well, will both reduce the amount of information to be processed (and so reduce the workload), and also go some way towards obtaining a higher level of understanding of the scene, by its very nature of eliminating the unimportant parts.

- Secondly, these features are matched between the frames. In the simplest and commonest case, two frames are used and two sets of features are matched to give a single set of motion vectors.

Most of the motion estimation algorithms make the following assumptions:

- Objects move in translate in a plane that is parallel to the camera plane, i.e., the effects of camera zoom, and object rotations are not considered.

- Illumination is spatially and temporally uniform.

- Occlusion of one object by another, and uncovered background are neglected.

There are two mainstream techniques of motion estimation:

1. Pel-recursive algorithm (PRA)

2. Block-matching algorithm (BMA).

- RAs are iterative refining of motion estimation for individual pels by gradient methods. BMAS assume that all the pels within a block has the same motion activity.

- BMAS estimate motion on the basis of rectangular blocks and produce one motion vector for each block.

- PRAS involve more computational complexity and less regularity, so they are difficult to realize in hardware. In general, BMAS are more suitable for a simple hardware realization because of their regularity and simplicity.

- Figure illustrates a process of block-matching algorithm. In a typical BMA, each frame is divided into blocks, each of which consists of luminance and chrominance blocks. Usually, for coding efficiency, motion estimation is performed only on the luminance block. Each luminance block in the present frame is matched against candidate blocks in a search area on the reference frame. These candidate blocks are just the displaced versions of original block.

- The best (lowest distortion, i.e., most matched) candidate block is found and its displacement (motion vector) is recorded.

- In a typical inter frame coder; the input frame is subtracted from the prediation of the reference frame. Consequently the motion vector and the resulting error can be transmitted instead of the original luminance block; thus inter frame redundancy is removed and data compression is achieved.

- At receiver end, the decoder builds the frame difference signal from the received data and adds it to the reconstructed reference frames. The summation gives an exact replica of the current frame. The better the prediction the smaller the error signal and hence the transmission bit rate.

Assignment- 6

1. What is the motion tracking?

- Motion tracking may refer to:

- Motion capture: the process of recording the movement of objects or people

- Match moving: a cinematic technique that allows the insertion of computer graphics into live-action footage with correct position, scale, orientation, and motion relative to the objects in the shot

- Video tracking: the process of locating a moving object over time using a camera

- Object tracking is an application of deep learning where the program takes an initial set of object detections and develops a unique identification for each of the initial detections and then tracks the detected objects as they move around frames in a video.

2. Explain applications of motion tracking.

The various applications are:

- Application of object tracking in unmanned aerial vehicles (UAVS). UAVS is commonly known as drones. Compared with the human eye, the drove has the advantages of stability and accurately capture. Therefore, drones using object tracking technology achieve robust can more results.

- Application of All object industries tracking in industrial automation. Intelligently, and object tracking is also widely used in industrial production. are developing

- Application of transportation. Today, an efficient transportation selection object tracking in intelligent system is vital for the in masses. Intelligent transportation, real-time monitoring and tracking of vehicles can be achieved using object detection and tracking technology

- Motion capture is a widely used technique in many fields like Biomechanics, sports, Gait, In Sound and Motion, MRI, Equine, Live events, Engineering, Films and games.

- In Biomechanics for the study of the structure and function of biological systems such as humans, animals, plants, organs, and cells by means of the methods of mechanics.

- In sports it is use to study all the actions of players. Physical limitations and movement optimization are of great interest to athletes, coaches, researchers and doctors. Motion capture allows us to learn more about injury mechanisms and prevention. It can also be used to improve a player’s technique for better results in various sports applications

- In Gait it is traditionally been studied subjectively through visual observations. By combining advanced measurement technology and biomechanical modeling, human gait can now be measured objectively. Gait analysis research and development is an ongoing activity, with new models and methods continually evolving. Today, physiotherapists, orthopedists and neurologists all use gait analysis to evaluate a patient’s status, treatment and rehabilitation.

- In Sound and Motion for the study of human motion.

- In MRI for Study of organs.

- In Equine for study of Horse Motion.

- In Live events for Presentations and Meetings.

- In Engineering for the study of mechanical designs and to observe the motion to improve the productivity.

3. Explain the motion tracking procedure.

- At first, the image to be processed needs to be input and the motion model also needs to be selected.

- The motion model is used to generate candidate sample information which may contain the target.

- The speed and quality of sample generation affect the speed and effectiveness of the entire tracking.

- Motion models typically include particle filtering and sliding windows. Particle filtering uses particle sets probabilities.

- This method approximates the probability density to represent finding a of the state space, and replaces the function by set random sample N propagating in integral operation with the sample mean.

- The sliding window generates a series of candidate samples on the calibration target frame through the cyclic matrix method.

- Next, Feature extraction is performed. Feature extraction is to find features in the candidate region which can uniquely identify the target. In object tracking, the quality of the feature has the most direct impact on tracking results.

- The observation model is used to determine whether the current sample is matching tracked object.

- The generative model and the discriminative model are divided according to the observation model. The generated model is tracked by searching for the object most similar to the target in the current area.

- This is a template matching process, where sparse representation is the most commonly used. Finally, a discriminant model is obtained through learning as a discriminator, and the discriminator is used to discriminate whether the current sample is the target.

4. Write a note an Iterated Estimation.

- Pose estimation algorithms utilize frames of a video feed from a moving camera to generate hypotheses for how the camera has moved over the course of each consecutive frame.

- Until now, algorithms used for pose estimation required the generation of up to five to ten hypotheses for how the camera was moving given the data in the video feed.

- These hypotheses were then scored by how well they fit the data, with the highest scoring hypothesis being chosen as the best pose estimate.

- Unfortunately, the generation of multiple hypotheses is computationally expensive and results in slower return time for robust nose estimation.

- Researchers have found a way to seed, or give hints to, an already used algorithm in computer vision by feeding it information in between each frame, thus greatly reducing the need for generating many hypotheses.

- "At each iteration, we hypothesis to seed the algorithm." use the current best The reduction of required hypotheses directly results in reduced computation time and complexity; "we show that this approach significantly reduces the number of hypotheses that must be generated and scored to estimate the pose, thus allowing real-time execution of the algorithm."

5. Explain Kalman Filtering.

- In statistics and control theory, Kalman filtering, also known as linear quadratic estimation (LQE), is an algorithm that uses a series of measurements observed over time, including statistical noise and other inaccuracies, and produces estimates of unknown variables that tend to be more accurate than those based on a single measurement alone, by estimating a joint probability distribution over the variables for each timeframe. The filter is named after Rudolf E. Kálmán, who was one of the primary developers of its theory.

- Kalman filtering has numerous technological applications. A common application is for guidance, navigation, and control of vehicles, particularly aircraft, spacecraft and ships positioned dynamically.

- The Kalman filter has long been regarded as the optimal solution to many applications in computer vision for example the tracking objects, prediction and correction tasks. Its use in the analysis of visual motion has been documented frequently; we can use in computer vision and open cv in different applications in reality for example robotics, military image and video, medical applications, security in public and privacy society, etc.

- In this, we investigate the implementation of a Matlab code for a Kalman Filter using three algorithm for tracking and detection objects in video sequences (block-matching (Motion Estimation) and Camshift Meanshift (localization, detection and tracking object)).

The Kalman filter is presented in three steps:

1. Prediction

2. Estimation (correction)

3. Update.

- The first step is a prediction for the parameters of the tracking and detection objects.

- The second step is a correction and estimation of the prediction parameters.

- The important application in Kalman filter is the localization and tracking mono-objects and multi- objects are given in results.

- This works presents the extension of an integrated modeling and simulation tool for the tracking and detection objects in computer vision described at different models of algorithms in implementation systems.

The kalman filter algorithm:

- In an object tracking algorithm, there are generally four steps: detection, location, association, and trajectory estimation [1, 2, 3].

- The algorithms are composed by three important modules: block matching and meanshift, camshift, Kalman filter.

- The Kalman filter is used in a wide range of technological fields.

- It is a major theme in automation and frame and signal processing.

- The Kalman filter "KF" is a set of mathematical equations which provide an efficient (recursive) computation of the means for estimation the state of a process.

- The KF is very powerful in several aspects: it supports estimates of past, present and even future states and it can do so even when the exact nature of the modeled system tracking and detection objects.

- The Kalman filter is a corrective predictor filter. In the tracking system objects, this filter looks at an object as it moves, that is, it takes information on the state of the object at the precise moment. Then, it uses this information to predict where the object is in the next frame.

- For this, it takes as input a measurement vector (position in x, in y, width and height of the object).

- In the tracking process, this filter looks at an object as it moves, that is it takes information on the state of the object at the precise moment.

- In the case of tracking an object in motion, the Kalman filter allows us to estimate the states of motion of the object (and therefore predetermine the areas of motion in the following frames with using the combination for three algorithms (block- matching, Camshift and Kalman Filter)) and thus adds robustness tracking objects.